DIVISION 08 - OPENINGS | SECTION 08 42 33

AUTOMATIC REVOLVING ENTRANCES

{CHOOSE 1: [AutoFlow], [MiniGrand], [Grand] REVOLVING DOOR SYSTEM}

Automatic Revolving Doors | F7.1 – F6.1  - F5.1

| Specification | Selection |
| --- | --- |
| Door ID | {door_id} |
| Product Name {Choose 1} | AutoFlow® Automatic Series 9331 – 3-Wing with Segmented Drum, Medium Stile [Diam10’] AutoFlow® Automatic Series 9330 – 3-Wing with Round Drum, Medium Stile [Diam10’] MiniGrand® Automatic Series 9331MG – 3-Wing with Segmented Drum [Diam 10’-12’] MiniGrand® Automatic Series 9330MG – 3-Wing with Round Drum [Diam 10’-12’] Grand® Automatic Series 9631 – 3-Wing Segmented Drum [Diam 12’-16’] Grand® Automatic Series 9630 – 3-Wing Round Drum [Diam 12’-16’] |
| Diameter | Per door schedule or TBD |
| Height (to top of roof) | Per door schedule or TBD |
| Design | Edit Section 2.02 I 1 & 2 |
| Finish | Edit Section 2.04 B 1 & 2 & 3 |
| Security Options | Edit Section 2.02 C 2 |
| Canopy | Edit Section 2.02 I 3 |
| Drum and Wings | Edit Section 2.02 J |
| CORE [ONLY 9630 &9631] | Edit Section 2.02 K |
| Voltage | Edit Section 2.03 A |
| Applicable codes | Edit Section 2.02 I 1 & 2 |

These specifications represent the exacting selection you have made for your project {PROJECT}, including door configuration, colors and options and depict design features that are commonly used. Horton Automatics will custom manufacture each product to these specifications. It is the customer’s responsibility to validate that each configuration is suitable for the intended application. All specifications and designs contained herein are subject to change without notice or obligation. Consult www.hortondoors.com’ spec desk at 1-800-531-3111 or specs@hortondoors.com for assistance.

DIVISION 08 - OPENINGS

SECTION 08 42 33 - REVOLVING DOOR ENTRANCES

PART I – GENERAL

1.01 SUMMARY

A. 	WORK INCLUDED: Furnish complete large diameter automatic aluminum revolving door system, as specified, that has been manufactured, fabricated, and installed to maintain performance criteria stated by manufacturer without defects, damage, or failure.

B. 	RELATED WORK:

1. 	Openings: Division 08, applicable sections.

2. 	Electrical: Division 26, applicable sections.

1.02 REFERENCES

A.  AMERICAN ASSOCIATION OF AUTOMATIC DOOR MANUFACTURERS (AAADM).

B.	AMERICAN ARCHITECTURAL MANUFACTURERS ASSOCIATION (AAMA) 101: Appendix Dissimilar Materials.

C. 	AMERICAN NATIONAL STANDARDS INSTITUTE (ANSI):

1. 	ANSI 156.27: Power and Manual Operated Revolving Pedestrian Doors

2. 	ANSI Z97.1: Safety Glazing Materials Used in Buildings - Methods of Test.

D. 	AMERICAN SOCIETY FOR TESTING AND MATERIALS (ASTM) B221: Aluminum-Alloy Extruded Bars, Rods, Shapes and Tubes.

E. 	THE ALUMINUM ASSOCIATION (AA): Aluminum Finishes Manual

F. 	INTERTEK, WARNOCK HERSEY (ETL): Testing Laboratory and Certification Agency joined with ETL SEMKO

G. 	UNDERWRITERS LABORATORY, INC. (USA & CANADA) UL 325: Electrical Door, Drapery, Gate, Louver, and Window Operators and Systems.

1.03 SUBMITTALS

A. 	SHOP DRAWINGS & PRODUCT DATA: Submit drawings and product data showing layout, profiles, product components including anchorage, accessories, finish and glazing details (where required).

B. 	QUALITY ASSURANCE AND CLOSEOUT SUBMITTALS: Submit the following:

1. 	Owner’s Manual & Warranty document as specified herein.

2. 	AAADM inspection compliance form completed and signed by certified AAADM inspector prior to doors being placed in operation as proof of compliance with ANSI A156.27.

1.04 QUALITY ASSURANCE

A. 	INSTALLERS QUALIFICATIONS: Installer shall be factory trained, certified by AAADM, and experienced to perform work of this section.

B. 	MANUFACTURER’S QUALIFICATIONS: Manufacturer to have minimum (5) five years successful experience in the fabrication of automatic doors of the type required for this project. Manufacturer capable of providing field service representation during installation, approving acceptable installer, and approving application method.

1.05 WARRANTIES

A. 	MANUFACTURER'S WARRANTY: Units to be warranted against defect in material and workmanship for a period of two years from the Date of Substantial Completion. Manufacturer's warranty is in addition to, and not a limitation of, other rights owner may have under Contract Documents.

B. 	DISTRIBUTOR'S WARRANTY: One-year warranty: Labor & transportation charges for defective parts replacement. Three-year Distributor’s Warranty for electronic speed control.

1.06 PROJECT CONDITIONS

Field Measurements: Verify actual dimensions/openings by field measurements before fabrication and record on shop drawings. Coordinate with fabrication and construction schedule to avoid construction delays.

1.07 DELIVERY, STORAGE AND HANDLING

A. 	ORDERING AND DELIVERY: Comply with factory's ordering instructions and lead time requirements. Delivery shall be in factory's original, unopened, undamaged containers with identification labels intact.

B. 	STORAGE AND PROTECTION: Provide protection from exposure to harmful weather conditions and vandalism.

# PART II – PRODUCTS

2.01 MANUFACTURER

Basis of design: Horton Automatics, a division of Overhead Door Corporation, shall manufacture automatic sliding door(s) of type(s) and size(s) specified on plans and door schedule. 4242 Baldwin Blvd, Corpus Christi, TX 78405. Toll Free: 361-866.6603 www.hortondoors.com. hortonautomatics@overheaddoor.com.

☐.[Substitutions:  Requests for substitution and product approval in compliance with the specifications must be submitted in writing and in accordance with the procedures outlined in Division 1, Section “Substitution Procedures”.  Approval of requests is at the discretion of the architect, owner, and their designated consultants.]

☒.[Substitutions:  Not Permitted.]

2.02 EQUIPMENT

Dimensions: {CHOOSE 1}

- Exact Dimensions:

Enter Width [opening width minus ½”]:

Enter Height [to top of the header}

- Follow door schedule

- MANUFACTURED DOOR UNITS: Horton Revolving door system with collapsing three-wing design. Units shall include operator, enclosure/drum with canopy, door wings/panels, center core, and pivot/bearing{CHOOSE 1}

  - AutoFlow® Automatic Series 9331 – 3-Wing with Segmented Drum, Medium Stile

  - AutoFlow® Automatic Series 9330 – 3-Wing with Round Drum, Medium Stile

  - MiniGrand® Automatic Series 9331MG – 3-Wing with Segmented Drum

  - MiniGrand® Automatic Series 9330MG – 3-Wing with Round Drum

  - Grand® Automatic Series 9631 – 3-Wing Segmented Drum

  - Grand® Automatic Series 9630 – 3-Wing Round Drum

B. 	OPERATOR: The operating mechanism shall be an adjustable power operator mounted within the enclosure canopy. The operator shall be supplied with 1/4 HP 1800 RPM high torque motor, gear box, and control panel and shall be ETL approved & tested to UL 325 Standard. Non-listed units shall not be acceptable.

1. 	The double seal, corrosion-proof, cast-iron gear case shall contain case hardened (60Rc) helical gears in synthetic, low temperature oil bath lubrication.

2. 	Motor drive shall be enclosed to prevent EMI noise from affecting the microprocessor control.

3. 	A 1 1/2" (38 mm) diameter solid steel center shaft shall serve as main linkage to connect the operator to the revolving door wings. The at-rest position of the door wings shall be adjustable to suit the traffic and desired function.

4. 	Optional: 90 VDC electric brake (for Park & Lock™ operation).

C. 	Master Control: The Microprocessor Master Control Panel (MCP) shall be a multiprocessor digital control system utilizing time-division multiplexed operations to provide precision motor-drive, constant sensor monitoring, and automated setup. An RS-232 port shall be provided for communication with external display (by others) to monitor door status, violations, and operation. All inputs will have an adjacent LED to verify input signals. Modular MCP shall be capable of exchanging the individual control components without having to

replace the entire system. Three (3) sets of programmable Form C contacts will be available for monitoring door functions or violations.

1. 	Parameters: Ninety-nine (99) adjustable functions shall be provided including (but not limited to):

a. 	Door speeds

b. 	Time delays

c. 	Reaction to, and force required for safety stops

d.	Safety stops before idle

2. 	Modes: Shall define how the door will be used. A key switch shall be provided for selecting operating modes. Operating modes provided shall include (but not be limited to):

a. 	Fully Automatic - Motion sensor activation for entry and exit

b. 	Exit only – door not locked but responds only to interior activation

c. 	Continuous run – after activation expires, door will continue to run at reduced speed until activation signal is received

d. 	Optional: Park & Lock™ – stops all activating signals so door will stop and lock (requires electric brake)

e. 	Optional: One-Way Security: {CHOOSE 1}

1. Totally secure, Card reader access for entry and exit

2. Card reader entry and free exit

3. Freewheel mode

4. None

3. 	Diagnostics: Shall be used to set-up and maintain unit via a wireless Local Remote Control (LCP) with seven segment diagnostic display from outside the door. Adjustments at the control or via a wired remote shall not be considered equal. The logic will perform a self-set up, not requiring technician intervention or changes to ensure a safe installation. Sixteen (16) diagnostic modes shall be provided including (but not limited to):

a. 	Calculating unit speed in RPM’s

b. 	Testing voice module

c. 	Global relearn (sets safety sensitivity to optimum level)

d. 	Checking motor voltage, current and optional brake voltage

e. 	Optional: (For One-Way Security) Single zone security mats will be able to discern whether the user is entering or leaving the secured zone of the door and communicate to MCP.

4. 	Voice Annunciator: Four (4) digital field programmable voice message shall be provided and shall be 100% solid state. Tape recorded message will not be accepted.

D. 	AUTOMATIC ACTIVATION:

1. 	Two motion sensors shall be placed at revolving door entrance to detect someone approaching door. This actuation shall cause door to revolve at the rate of 3-5 RPM (adjustable) for one complete turn after actuating signal is removed. Door will then slow then stop at the ‘at-rest’ position.

2. 	Two momentary contact switches with push button shall be placed at entrance to the door. The switch mounting plate to read "Push Button to Slow Door". Pushing this button will cause door to revolve at 1-3 RPM for a selected amount of time. A digital voice annunciator will say "Door in Slow Speed, Do Not Push." Slow speed operation will be protected by a tracking governor only allowing more than 1 RPM over the adjusted speed. Note: Door can annunciate “Caution, door speed will increase” before resuming normal run speed (selectable)

E. OPTIONAL: TRAFFIC CONTROL FUNCTION – SELECTABLE DAY/NIGHT OPERATION: To provide a barrier free exit to the non-secure area while maintaining a restricted entrance to the secure area.

1. 	One-Way Security: Traffic shall be allowed to pass unrestricted from one direction and controlled from opposing direction. A key switch shall be used by security personnel to gain access to secure areas and to switch to manual (freewheel) operation in either direction, as well as, to return systems to normal operation. Free access side shall be supplied with motion sensor to set door in motion. An actuation signal shall cause door to revolve at 2 to 4 RPM's depending on door size (adjustable) for a complete turn after actuating signal is removed, door will slow and stop at the at-rest position. Any attempt of entry from controlled access side during this operation shall be detected. The only way passage will be allowed from controlled access side is by key switch.

2. 	Day Operation/Non-Security Mode: A double pole, double throw switch signal by others (dry contact) shall cause control to operate as a two-way free access in both directions. Actuation shall be from motion sensors (optional), activated by logic software. Any mat signal shall cause door to turn to avoid entrapment.

F. OPTIONAL: SECURITY FUNCTION: Area detection mat shall be placed inside door where traffic passage is to be controlled. Electric brake shall be a fail-safe design with battery backup to provide full operation for 2 hours in the event of power failure. Brake shall have enclosed construction with sealed ball-bearings and no backlash. Control shall have integral anti-entrapment logic built in to eliminate any possible entrapment.

1. 	Any attempt of passage into unauthorized area shall cause door to ramp to a stop, lock and a solid-state voice annunciator will say "Security Violation, Please Exit" (Tape recorded systems will not be acceptable.).

2. 	Door will then rotate to a position allowing secure area to be cleared and proceed forward in slow speed for 1/3 revolution and then resume normal speed.

G. 	COLLAPSING MECHANISM: Door design shall be equipped with magnetic breakaway and completely bookfold in case of emergency as required by Codes.

1. 	Electromagnets shall be capable of holding with more than 1000 lbs. (4400 N) min. force to maintain system integrity and hold door wings in their respective positions under normal conditions.

2. 	Alarm Circuit: Shall be supplied and be connected to the building's alarm system. Activation of the fire alarm, smoke detector or remote emergency button shall remove power to the door's electromagnetic locks and allow the wings to completely book fold in case of emergency as required by Codes.

3. 	Emergency Switch: Shall be provided and mounted near the door and shall be labeled "Break Glass to Release Magnetic Locks." Wings shall maintain their respective positions when power is released but can be broken away into true book-fold position with 80-130 lbs. (355-578 N) of force (adjustable).

H. 	SAFETY: The following safety features shall be provided as per ANSI A156.27:

1. 	Entrapment Protection (‘Fail-Safe’ during power loss): Door wings shall be free to rotate manually in either direction when power is removed. Magnetic locks will disengage when power is removed.

2. 	Alarm Contacts: Shall be provided to activate remote signal (by others) when door has loss of power.

3. 	Torque Limiting: Shall be provided via a back pressure sensing circuit adjusted so that 20-30 lb. (67-133 N) back pressure will stop door’s rotation.

4. 	Cushioned Wall Safety Edges: Shall be provided at throat entrances to stop door’s rotation when depressed for more than 1/4 of a second.

5. 	Bump-to-Idle™ (Safety Stop Before Idle): If back pressure circuit or safety edge is activated during normal operation, door will go into the IDLE MODE. Door wings shall be free to rotate manually in either direction. After a forward manual push, the door will then restart at reduced speed and gradually accelerate to normal speed.

6. 	VistaScan™: Sensor shall be mounted at top of door wing and detect a 28” (710mm) minimum high person or equivalent in rotating path 10” (254mm) minimum from the face of the wing and shall cause door to slow to maximum allowed kinetic energy speed. The sensor shall provide a minimum active area for the width of wing less 15” from center shaft and 5” from outer edge of outer stile.

7. 	ToeGuard™: Cushioned contact switch sensor shall be mounted at wing bottom rail and be active in the rotating path of the wing. It shall be active within 2” (50mm) from outer edge of the outer stile end and 6” (150mm) from the center of the door and not higher than 4” (100mm) from the finished floor. Contact switch shall require no more than 10 lbf. (54N) pressure to activate. Upon receipt of signal, door shall stop rotating.

8. 	EntryGuard™ Sensor: An infrared device will be tied to door rotation and stop the door any time an object is detected when the display case is within 24” (612mm) adjustable of the entrance throat post. This distance shall be adjustable within software and its function selectable to slow or stop depending on customer choice.

I. 	ENCLOSURE: Shall be extruded aluminum and glass/glazing material and shall be constructed maintaining proper clearances and weather seal.

1. 	{CHOOSE FOR SERIES 9331, 9331MG, 9631} Segmented® Design

2. 	{CHOOSE FOR SERIES 9330, 9330MG, 9630} Round Design

3. 	{CHOOSE IF ROUND CANOPY IS SPECIFIED} Canopy: Standard canopy be shall round with curved aluminum 1/8" (3) facia panels and shall be minimum 24” (610mm) high. Interior ceiling shall be 3/4” thick plywood laminated with .060” (1.5mm) aluminum to match door finish. Canopy to include eight ceiling lights with energy efficient lighting systems with flush lens.

Canopy options shall include: {CHOOSE THOSE THAT APPLY}

a. 	Extended canopy.

b. 	Segmented® canopy on Segmented® enclosure.

c. 	Cropped canopy.

d. 	Exterior roof fabricated from .090" (2mm) anodized aluminum.

e. 	Backlit fluorescent signage placed on one or both sides of canopy.

J.  PANEL/DOOR WING: Shall be aluminum with perimeter weather-stripping utilizing affixed sweeps shall ensure weather seal. An intermediate, horizontal muntin bar shall be furnished on each wing for safety and division of glass. {CHOOSE 1}

- Series 9330 & 9331: Narrow

- Series 9330 & 9331: Medium

- Series 9330 & 9331: Wide

- Series 9330MG & 9331MG & 9630 & 9631: Medium

K. 	{CHOOSE FOR 9630 & 9631 ONLY} CORE: Rotating central portion of door unit shall have top and bottom hydraulic closers for each wing to cushion emergency wing collapse. Core shall be extruded aluminum with glass/glazing material and perimeter weather seal and shall include metal apron with two fixed display panels and one hinged access panel. Access panel shall have top and bottom cam locks. Weight limit for Display floor shall be 200 lbs. (90 kg.)

L. 	HARDWARE: Provided shall include:

1. Locks with five pin cylinders and concealed bolts provided on two door wings.

2. Cam locks for Display Core access panel.

3. Bottom pivot/bearing: Surface mounted with no excavation below floor line required.

4. Center Shaft: 2” (51mm) diameter steel shaft with connections to operator and bottom pivot/bearing.

5. Hydraulic Closers: Shall be double acting and closing speed shall be adjustable.

2.03 RELATED WORK REQUIREMENTS

A. 	ELECTRICAL; {CHOOSE 120VAC or 240VAC] 60 cycle, single phase, 20 amp in conduit routed to canopy. Remote switch locations shall require routing of low voltage class ll wiring in conduit routed to canopy. Remote switch locations shall be

predetermined and wired before installation.

B. 	GLASS AND GLAZING: Glass stops, glazing vinyl and setting blocks for field glazing as per Safety Glazing standard ANSI Z97.1.2. General contractor to coordinate acquisition of glass, not already provided by manufacturer, in thickness and type in accordance with manufacturer's recommendations for prescribed design.

1. 	{CHOOSE 1}  Door Wing Glazing: (provided by others)

a.   Standard flat safety glass in clear, 1/4” (6mm) tempered

b.	1/4” (6mm) clear tempered glass

c.	7/16” (11mm) laminated glass with Solar Blue, Grey or Bronze tint

d.	7/16” (11mm) Starphire Low Iron laminated glass

e.	9/16” (14mm) clear laminated glass

f.	9/16” (14mm) Starphire Low Iron laminated glass

2. {CHOOSE FOR 9630 &9631 ONLY} Display Core: Standard material will be flat safety glass in clear, 1/4” (6 mm) tempered.

3. {FOR 9331, 9331MG & 9631 - CHOOSE 1} Segmented® Enclosure Glazing: (provided by others)

a.   Standard flat safety glass in clear, 1/4” (6mm) tempered

b.	1/4” (6mm) clear tempered glass

c.	7/16” (11mm) laminated glass with Solar Blue, Grey or Bronze tint

d.	7/16” (11mm) Starphire Low Iron laminated glass

e.	9/16” (14mm) clear laminated glass

f.	9/16” (14mm) Starphire Low Iron laminated glass

4. 	{FOR 9330, 9330MG & 9630 - CHOOSE 1} Round Enclosure Glazing: (provided by factory) Standard material will be curved safety glass in clear, 7/16” (11mm) laminated. Optional glazing shall include:

a.	1/8” (3mm) aluminum

b.	1/4” (6mm) clear tempered glass

c.	7/16” (11mm) laminated glass with Solar Blue, Grey or Bronze tint

d.	7/16” (11mm) Starphire Low Iron laminated glass

e.	9/16” (14mm) clear laminated glass

f.	9/16” (14mm) Starphire Low Iron laminated glass

2.04 MATERIALS, FINISHES AND FABRICATION

A. 	EXTRUDED ALUMINUM: ASTM B221, 6063-T5 alloy and temper, anodized:

1. 	Structural Drum/Enclosure Sections: Minimum 1/8" (3mm) thickness.

2. 	Structural Panel/Door Wing Sections: Commercial grade.

B. FINISH {CHOOSE 1, 2 OR 3}

1. Anodize finish: {CHOOSE 1}

- 204-R1 Clear: Arch. Class 1 Clear Anodized Coating, AA-MI2C22A31.

- Champagne: Arch. Class 1 Anodized Coating, AA-MI2C22A44.

- 311-R1 Light Bronze: Arch. Class 1 Anodized Coating, AA-MI2C22A44.

- 312-R1 Medium Bronze: Arch. Class 1 Anodized Coating, AA-MI2C22A44.

- 313-R1 Dark Bronze: Arch. Class 1 Anodized Coating, AA-MI2C22A44.

- Paint Coating: {CHOOSE 1}

- Powder Coat: 100% V.O.C. free fluoropolymer resin-based. Color: {color}.

- Wet Paint. Color: {color}

- Clad finish WINGS & ROUND DRUM ONLY: {CHOOSE 1}

- Stainless Steel #4 (Satin Finish): Architectural Satin Finish Clad Coating.

- Stainless Steel #8 mirror finish (Reflective): Architectural Reflective Clad Coating.

- Bronze #4 (Satin Finish): Architectural Satin Finish Clad Coating.

- Bronze with polish, non-directional finish: Architectural Non-Directional Polished Clad Coating.

- Brass #4 (Satin Finish): Architectural Satin Finish Clad Coating.

- Brass with a polished, non-directional finish: Architectural Non-Directional Polished Clad Coating.

- Muntz metal (brass alloy): Architectural Clad Coating.

C. 	DOOR WING CONSTRUCTION:

1.	Corner block type with 3/16” steel backup plate construction, mechanically secured with minimum of four hardened steel screws. Sash consists of snap-in glass stops, snap-in glazing beads and vinyl gaskets.

2.	Weather-stripping material captured in extruded aluminum door panel. Surface applied self-adhesive weather-stripping not acceptable.

D. 	ENCLOSURE AND CORE CONSTRUCTION: Butt joints, mechanically secured by means of screws and formed aluminum corner brackets.

PART III - EXECUTION

3.01 EXAMINATION

Site Verification of Conditions: Installer must verify that base conditions previously installed under other sections are acceptable for product installation according to with manufacturer's instructions. Notify the Contractor in writing of conditions detrimental to the proper and timely completion of work. Do not start work until all negative conditions are corrected in a manner acceptable to the installer and manufacturer.

3.02 INSTALLATION

A. 	GENERAL: Install door unit(s) plumb, level and true to line, without warp or rack of enclosure. Drum enclosure can be center connected to adjacent construction or 'throat connected' (at entry vertical rails). Allow for 1/4" shim space each side and at canopy. Anchor in place. Unit is free-standing with no additional support required. Adjacent construction not to bear down on revolving door.

B. 	DISSIMILAR MATERIALS: Comply with AAMA 101, Appendix Dissimilar Materials by separating aluminum materials and other corrodible surfaces from sources of corrosion or electrolytic action contact points.

C. 	WEATHER-TIGHT CONSTRUCTION: Install enclosure with joint filler or gaskets and sealant. Coordinate installation with wall flashings and other components of construction.

D. 	ELECTRICAL: General or electrical contractor to install all wiring to optional equipment on a separate circuit breaker routed into canopy.

3.03 CLEANING, ADJUSTMENT AND PROTECTION

A. 	CLEANING: After installation, installer to take following steps:

1. 	Remove temporary protection of adjacent work areas. Remove construction debris from construction site and legally dispose of debris.

2. 	Repair or replace damaged installed products. Clean product surfaces and lubricate operating equipment for optimum condition and safety.

B. 	ADVISE CONTRACTOR: Of precautions required through the remainder of the construction period, to ensure that doors will be without damage or deterioration (other than normal weathering) at the time of acceptance.

END OF SECTION

Horton Automatics reserves the right to make product improvements and change specifications without notice.